- 3D numerical modelling for interpreting topographic effects in rocky hills

- for Seismic Microzonation: the case study of Arquata del Tronto hamlet

- Ilaria Primofiore1*, Julie Baron2, Peter Klin2, Giovanna Laurenzano2, Cristina Muraro3, Franco

- Capotorti3, Marco Amanti3, Giovanna Vessia1

- 1 University “G. d’ Annunzio” of Chieti-Pescara, Department of Engineering and Geology, Via dei Vestini

- 31, 66100 Chieti, Italy (ilaria.primofiore@alumni.unich.it; g.vessia@unich.it)

- 2  National Institute of Oceanography and Applied Geophysics (INOGS), Borgo Grotta Gigante 42/C,  34010

- Sgonico, Trieste, Italy (jbaron@inogs.it; pklin@inogs.it; glaurenzano@inogs.it)

- 3  Italian Institute for Environmental Protection and Research (ISPRA), Via Brancati, 48, 00144 Roma,  Italy

- (cristina.muraro@isprambiente.it; franco.capotorti@isprambiente.it; marco.amanti@isprambiente.it)

- * Corresponding author. E-mail address: ilaria.primofiore@alumni.unich.it

- ABSTRACT

- The 2016-2017 seismic sequence in the Central Apennines (Italy) demonstrated, once again, the

- relevant role of the differentiated seismic effects related to different features in complex  geological

- contexts  at  a  short  distance  (i.e.  soft  sediment-filled  valley,  hilly  relieves,  complex       buried

- geometries).  The heavy  damages  of old  settlements  located  on  the  top  of a  rocky hill,  such as

- Arquata del Tronto hill, raised the question on the main role of three-dimensional movements of

- asymmetrical isolated rocky reliefs in generating disruptions during the seismic shaking. In order to

- shed light on these effects, the writing authors analysed the seismic response of a digital 3D   model

- of  the  Arquata  del  Tronto  hill,  that  was  constructed  by  considering  the  topography  and   the

- underground  geological  settings  of  the  area.  The  seismic  response  was  evaluated  through 3D

- numerical  simulations  of  seismic  waves  propagation  with  the  spectral  element  method  for the

- period range 0.1-1 s, that covers the typical resonance frequencies of the buildings involved in the

- studied case. The obtained results explain the recorded strong motion at Arquata site and are

- consistent with the observed heterogeneous damage distribution. This study evidences the need   for

- accurate 3D models in the evaluation of the seismic response of ridges with topographic and

- geological asymmetries and demonstrates that the common usage of 2D simplified simulations to

- approximate  the  3D  seismic  behaviour  is  not  correct.  Additionally,  the  study  points  out    the

- paramount role that the characterization in engineering - geological terms plays in the numerical 3D

- simulation of the seismic site response. Thus, in microzoning studies, simplifications related to both

- dimensional and geo-litho-seismic characterizations should be avoided.

- Keywords:   Seismic Microzonation studies, topographic effects, 3D response analysis,   2016-2017

- Central Italy seismic sequence, Arquata del Tronto.

- Highlights:

- ●	3D seismic response analyses of the steep asymmetric rocky ridge of Arquata del Tronto

- village;

- ●	Results evidence the 3D kinematics due to a ridge asymmetric shape;

- ●	2D simulations  are not  able to  capture the damages  observed  on  the Arquata  del Tronto

- hill;

- ●	Reliable 3D models of ridges must be supported by adequate geological and geophysical

- data.

42

# 1. Introduction

- From August 2016 to January 2017, a seismic sequence struck the Central Apennines (Italy),

- involving tens of municipalities and producing heavy damages, as well as about three hundred

- casualties. The first shock on 24th August 2016, at 1:36 UTC, occurred near Amatrice and Accumoli

- with Mw 6.0 and hypocentral depth of 8 km and was followed by an Mw 5.4 event at 02:33 UTC

- close  to  Norcia  (reference  website:  itaca.mi.ingv.it).  They  caused  extensive  and       irregularly

- distributed damage over the area (Galli et al., 2016). Subsequent shocks (on 26th  October, Mw 5.4 at

- 17:10 and Mw  5.9, at 19:18 UTC, on 30th  October, Mw 6.5 at 5:40 UTC, and finally on 18th January

- 2017 four shocks with Mw ranging from 5.0 to 5.5) aggravated the level of damage (Graziani et al.,

- 2019; Lanzo et al. 2019). The damage spatial pattern indicates that strong local modifications of

- earthquake ground motion (i.e. site effects) took place in the area (Sextos et al., 2018) due to

- stratigraphic and topographic effects. As a matter of fact, in the aftermath of the first mainshock   of

- the 2016-2017 sequence, the macro-seismic surveys reported an EMS intensity 8-9 at Arquata del

- Tronto old town located along the rocky ridge. On the other hand, 7 EMS intensity was reported   at

- the Borgo hamlet located in the alluvial valley at the foothill (Graziani et al., 2019; Lanzo et al.

- 2019).	This case study represents a typical example of complex seismic site effects in Central

- Apennines (Amanti et al., 2020).

- The study presented hereinafter intends to shed light on the occurrence of the so-called “atypical

- topographic effects” in the case of asymmetric rocky ridges. Typical topographic effect defines   the

- response of homogeneous rock hills when the morphology is the main factor that contributes to  the

- ground motion amplification. Instead, the atypical topographic effect is referred to several factors

- (that are structural and    geological heterogeneities) concurring to the definition of the site response

- (Burjanek  et  al.,  2014).  This  latter  contribution  may  imply  higher  amplification  values,        a

- broadening of the amplification frequency range and a kinematic pattern in the horizontal plane -   a

- parasitic twisting motion- which can severely increase the damage level on buildings. To this    aim,

- we adopted a well-established 3D numerical modelling method for seismic waves propagation,  that

- is the Spectral Element Method (SEM), to investigate the site response of Arquata del Tronto hill

- and its surroundings. The open-source SPECFEM3D-Cartesian code (Peter et al., 2011) was used to

- perform numerical simulations.

- A fundamental step for addressing 3D numerical simulations consists of setting-up an accurate   3D

- model of the subsoil geological structure using a computer-aided 3D geological modeller. This  step

- was	accomplished	through	the	commercial	software	GeoModeller	(https://www.intrepid-

- geophysics.com/product/geomodeller,   last   access:   7th     January   2020),   using   geological  and

- geomorphological data that were collected in the framework of Seismic Microzonation studies of

- the Arquata del Tronto municipality (ISPRA, 2017).

- In the following, section 2 is devoted to the description of some other authors relevant studies in

- seismic topographic effects; section 3 introduces the study area, summarises the observed damage

- pattern at Arquata del Tronto and Borgo hamlets and describes the construction of the 3D digital

- geological model of the study area. Section 4 illustrates the methods we employed for the 2D and

- 3D numerical simulations. Finally, we illustrate the results and an explanation of the observed

- differentiated damage pattern in section 5.

84

# 2.  State of the art in the study of topographic effects

- The term “topographic effect” defines seismic motion modifications that occur as the topographic

- surface reflects, diffracts, scatters or refocuses the incident seismic waves. Numerous cases of

- seismic damage variability in areas with notable relief (Davis and West, 1973; Çelebi, 1991; Vessia

- et  al.,  2013;  2016) suggests  that  topographic effects  may have  a significant  impact  on   ground

- motion in the frequency range, where most buildings are vulnerable. While researchers have so   far

- conducted quite extensive theoretical and experimental investigations of topographic site effects

- (see Paolucci (2002) and Massa et al. (2014), among others, for a rather exhaustive overview of  the

- topic) the development of a robust quantification approach is still at its early days. Some authors

- (Massa et al., 2014; Barani, 2014) made a distinction between typical and atypical effects. This

- latter distinction has been systematically studied by field observations and numerical simulations

- during the recent NERA European project (Burjanek et al., 2014). As a synthesis of a large  number

- of analysed  case studies,  the “topographic  effects” turn out to be a complex  combination of    two

- contributions:  the  topographic  features  and  the  local  sub-surface  conditions  (mechanical    and

- geometrical characters) of soils and rocks. Researchers found that all but A Eurocode 8 soil  classes

- suffer  motion  amplification  at  site-dependent  frequencies  and  along  with  selected    directions.

- Pischiutta et al. (2018) performed a systematic analysis using stations of the Italian seismic network

- to derive an indication about the occurrence of topographic amplification. They did not obtain    any

- systematic  relations  between  seismological  evidence  and  morphology,  concluding  that     other

- features in the subsoil should have a prevailing role in producing directional effects, such as   large-

- scale  open  cracks  (Marzorati  et  al.  2011;  Moore  et  al.  2011;  Burjanek  et  al.  2012)     and/or

- microcracks in fractured rocks (Pischiutta et al., 2013, 2015).

- 2.1. Experimental studies

- The  topographic  effects  have  been  extensively  investigated  through  field  measurements     and

- numerical simulations. From experimental studies, the polarization in the slope seismic response

- was at first highlighted by Spudich et al. (1996), who found that seismic motion on a ridgetop is

- polarized  perpendicularly  to  its  elongation  as  an  effect  of  wave  scattering.  This  effect     was

- confirmed by many other authors (e.g. Burjanek et al., 2014 and Del Gaudio and Wasowski,  2011).

- This topic has been studied both in soil and rocky slopes through ambient noise HVSR, standard

- spectral ratio SSR and arrays of strong earthquake seismographs. Results pointed out the occurrence

- of stronger spectral amplifications along the maximum slope direction in soil slopes (Del Gaudio

- and  Wasowski,  2011).  In  rocky  slopes,  using  several  different  experimental  and      numerical

- techniques, Burjánek et al. (2014) found the maximum ground motion amplification was oriented

- along the steepest slope. This latter effect is often combined with the heterogeneity of the  fractured

- slope  surface  into  a  complex  behaviour  that  cannot  be  thoroughly  understood  if      numerical

- simulation models do not consider them all (Kazemnia Kakhki et al., 2020). Using observational

- data from a dense array of strong earthquake seismographs deployed along the Lengzhuguan   slope

- (China),  He  et  al.  (2020) studied  its  seismic response to  the 2008  Wenchuan  earthquake. They

- highlighted that the seismic ground motion on the ridgetop was polarized perpendicularly to its

- elongation with an amplification factor higher than 10 in the frequency band ranging between 1.5

- and 2.5 Hz, where the rocky slope was covered by a loose deposit layer of about 30 m depth.

- Sepulveda et al. (2005) highlighted that during the 1994 Northridge earthquake (6.7 Mw) significant

- topographic amplifications have been experienced along rocky ridges in California and measured by

- local accelerometric stations: the peak accelerations measured at the crests were more than 3   times

- larger than the ones measured at the foothill. Finally, Zhang et al. (2018) measured the seismic

- response of the Xishan Park ridge under the 2008 Wenchuan    earthquake using the SSR technique:

- the  field  results  showed  that  topography  and  local  presence  of  soft  materials  on  the   surface

- significantly increases the ground motion. The maximum amplification factor, as large as 4, was

- observed at the hilltop within the frequency range of 1–10 Hz.

- 2.2. Numerical simulations

- Concerning  the  numerical  simulation  experiences  performed  in  the  literature  on  seismic slope

- response, the first 2D ones were performed in the ‘80s on simplified ridge models, with a constant

- velocity  through  the  section  (see  Geli  et  al.,1988  and  the  references  therein).  These    studies

- explained slope amplification in terms of wave scattering by the convex shape of topography and

- the amplified resonance frequency depending on the hill dimension and its velocity. The growing

- number of available data has recently permitted several back-analyses of 2D and 3D ridge models

- with numerical methods. Ding et al. (2020) studied the effects of surficial soil properties (Vs,

- thickness and material) on topographic amplification, based on a 2D slope model with   horizontally

- stratified soil layers, with nonlinear elastoplastic behaviour. The results confirmed that the extent of

- the topographic effect gradually increases with Vs values decreasing. Gischig et al. (2015) dealt

- with  the seismic response of unstable deep  rocky slopes.  With  a series  of 2D numerical   models,

- they explored the role of seismic amplification to increase the amplitude of the co-seismic slope

- deformation,  considering  Vs,  topography  and  fractures.  2D  results  show  that  3D  modelling is

- needed to understand the co-seismic slope deformations. Furthermore, Paolucci (2002) studying

- four  isolated,  irregular  and  densely  inhabited  relieves  in  Italy  evidenced  that  2D     numerical

- simulations underestimate the amplification factors for all the topographic categories listed by the

- Eurocode  8.  This  is  true  in  the  cases  of  narrow  and  large  isolated  cliffs,  ridges  with    crest

- significantly shorter than the base and the average slope angles both lower and higher than 30°.

- Additionally, Paolucci evidenced strong 3D effects that cannot be approximated by 2D seismic

- response analyses. A method for predicting topographic amplification was presented by Maufroy et

- al. (2015), who uses the curvature of the surface, defined as the second spatial derivative of the

- elevation map, as a proxy for the topographic amplification. This method was used with   promising

- results  to  predict  the  observed  damages  at  Amatrice,  after  the  24th   August  2016    earthquake

- (Maufroy et al., 2018). Lately, Luo et al. (2020) investigated the topographic effects at the rocky

- Mt.  Dong and  Weigan  hill  in Sichuan  Province (China) using monitoring data and  2D and     3D

- numerical modelling performed through Udec 2D and 3D codes. They measured amplifications    as

- large  as  5-6  times  the  input  motion  that  was  predicted  by 3D numerical  models  only if   they

- reconstructed the complex combination of the topographic and subsurface geological setting of   the

- area.  In  general,  numerical  simulations  of  slope  seismic  response  underestimate  the measured

- amplification factors (Massa et al., 2014, Lovati et al., 2011, Geli et al., 1988, Spudich et al.,   1996,

- Paolucci et al., 1999). This discrepancy has been attributed to a common over-simplification of   the

- geometries, i.e. 2D instead of 3D ridge models, that demonstrated to be unable to reproduce, in

- several cases, the complexity of the seismic wavefield modifications (Massa et al. 2014). Hence,  in

- the present work, both 2D and 3D numerical analyses have been performed at Arquata del Tronto

- steep rocky ridge, based on a detailed reconstruction of geo-lithological and seismic subsoil  model,

- to understand its complex seismic response.

# 3.  The case study of Arquata del Tronto

- Arquata del Tronto (42.7725 N, 13.2963 E) is an Italian municipality set in the Marche region. It  is

- located in the Central Apennine belt, at the base of the South-East flank of Vettore Mount. While

- the Old Town is located on a small rocky hill, the municipality has an area of 92 km2, consisting  of

- 13 hamlets (Fig. 1). Among these, the hamlet of Borgo has been recently urbanized, at less than 1

- km from the Old Town, in the valley beneath it. In this work, the following acronyms were used for

- the sites of major interest (Fig. 2): site STA, for the Old Town of Arquata del Tronto; site STB,  for

- the hamlet of Borgo; site STC is located in the Western part of the Old Town, at the base of a

- medieval Castle, almost 200 m uphill from the Old Town.

- Fig. 1. Epicentre location map of several events generated by the 2016-2017 Central Italy seismic sequence.

- 3.1. Recent seismicity and differentiated damages occurred

- The town of Arquata del Tronto, being in the centre of the Apennine chain, suffers from the

- influence  of  numerous  nearby  faults,  even  a  few  kilometres  away.  The  Database  of     Italian

- Seismogenic  Sources  (DISS  working  group,  2018)  shows  many  faults  in  the  surroundings  of

- Arquata del Tronto: they were, and potentially will be, sources of destructive earthquakes for the

- municipality. The proximity and the number of active faults determine a very high seismic risk.   In

- Table  1,  historical  earthquakes  that  shook  Arquata  del  Tronto  over  time  with  a macroseismic

- intensity	on	Mercalli-Cancani-Sieberg	scale	(MCS)	higher	than	6	are	listed

- (https://emidius.mi.ingv.it/CPTI15-DBMI15/).  Thus,  this  territory  has  been  mostly  affected  by

- seismic events generated in the same areas as the 2016-2017 seismic sequence.

| 198 | Table 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Intensity to Arquata del Tronto | Date | Epicentre | Epicentral intensity | Mw |
|  | 9 | 1703-01-14 | Valnerina | 11 | 6.9 |
|  | 7-8 | 1730-05-12 | Valnerina | 9 | 6.0 |
|  | 7 | 1916-07-04 | Monti Sibillini | 6-7 | 4.8 |
|  | 6-7 | 1972-11-26 | South Marche | 8 | 5.5 |
|  | 6-7 | 1950-09-05 | Gran Sasso | 8 | 5.7 |

- Seismic events that caused macroseismic effects higher than 6 MCS in Arquata del Tronto.

- Graziani et al. (2019) performed a throughout and exhaustive study of the damages caused by the

- seven events whose magnitude was larger than 5.4 (the first occurred on 24th August 2016, Mw 6.0,

- with the main event felt on 30th October 2016, Mw 6.5).

- The  authors  described  the  damages  through  the  European  Macroseismic  Scale  intensity   IEMS

- (Grünthal,  1998),  reporting  the  spatial  distribution  of  the  damage  magnitude  for  different

- vulnerability classes (VC) of buildings. VC decreases from A that means high vulnerability (simple

- stone masonry with irregular texture and a generally bad state of preservation) to D meaning low

- vulnerability  (reinforced  concrete  structures  with  anti-seismic  design  and  a  good  state  of

- preservation). At site STA, the damage was equally distributed among buildings of vulnerability

- classes A and B, whereas class B suffered major damage at site STB.

- After the first shock, at site STA, most buildings (72%) were affected by structural damage and few

- buildings underwent total destruction (14%). This behaviour has been recognized on both VC A and

- B, especially located on the steep and external portion of the hill. The behaviour of vulnerability

- class A corresponded to 9 IEMS, while class B experienced 8 IEMS.

- At site STB, slight and medium damage was observed in buildings of vulnerability class A and B,

- and the damage scenario is as high as IEMS. In the aftermath of the last earthquake (30th  October), 9-

- 10 IEMS was assessed at site STA, while at site STB the damage reached 8-9 IEMS. The differences in

- the  accumulated  damage  pattern  at  Arquata  del  Tronto  and  Borgo   hamlets  are  caught       by

- Laurenzano et al. (2019), Lanzo et al. (2019) and Giallini et al. (2020) studies. They suggested  that

- 3D topographic amplification conditioned the local site effects. Accordingly, the writing authors

- reconstructed 2D and 3D numerical models of the area, to investigate possible kinetic and inertial

- effects induced by the seismic wave propagation.

- 3.2. Geological and seismotectonic features

- The study area is located on the outer edge of the Central Apennines at the foot of the Olevano-

- Antrodoco-Sibillini Mountains thrust, in the Western outcropping area of the Messinian turbidite

- deposits of the Laga Formation. According to the Seismic Microzonation studies of the   Macroarea

- 1, Arquata del Tronto and Montegallo (map at 1:5.000 scale by ISPRA (2017)), the siliciclastic

- deposits  occurring  at  Arquata  del  Tronto  correspond  to  the  Campotosto  member  of  the Laga

- Formation  (LAG4).  The  member  is  subdivided  into  many  units        characterized  by  different

- arenite/pelite (A/P) ratio. In the analysed area, four lithofacies associations of this member crop out:

- the arenaceous association (LAG4c), the arenaceous-pelitic association    I (LAG4d), the arenaceous-

- pelitic association II (LAG4b) and the pelitic-arenaceous association (LAG4e). The subsoil   geology

- of Arquata del Tronto consists of a repeated alternation of the previous four lithofacies. Other

- details can be found in Appendix A. A detailed geological and geomorphological map of the area

- was drawn for the level 3 Seismic Microzonation studies of the Macroarea 1 Arquata del Tronto and

- Montegallo	(AP)	(ISPRA,	2017)	and	it	can	be	downloaded	for	free

- (http://www.halleyegov.it/c044006/hh/index.php). A simplified and cut out version of the map is

- shown in Figure 2, together with the related geological cross-sections (Figs. 3, 4, 5) investigated  in

- this study.

- The beds  are oriented  towards  the West  with high  angle slope.  From  ISPRA  surveys, they were

- classified as a reverse side according to the hypothesis that they are part of a large overturned

- anticline fold, developed in the hanging-wall of a thrust. This interpretation also derives from the

- work of Centamore et al. (1991). The original geological cross-sections have been slightly modified

- by the ISPRA geologists who processed them to be give more details for 3D modelling purpose.

- The main change from the original version concerns the concavity of the beds of the fold located

- below the old town of Arquata del Tronto, to follow-up on overturned fold hypothesis. Accordingly,

- a high degree of fracturing on the surface was detected, believed to be related to successively

- normal faulting.

- The local bedrock is mostly covered by continental deposits, consisting of calcareous detritus mixed

- to arenaceous boulders, in a sandy silt matrix, alluvial and alluvial terraces deposits, of variable

- thickness, based on the genesis of the deposit and the morphology of the accumulation area.

- Fig. 2. Simplified extract of the geological and geomorphological map used for the level 3 Seismic

- Microzonation studies of the Macroarea 1 Arquata del Tronto and Montegallo (AP), with section names and

- acronyms used for the sites of major interest.

- Fig. 3. Geological section 1-1’ from Figure 2.

- Fig. 4. Geological section 2-2’ from Figure 2.

- Fig. 5. Geological section 3-3’ from Figure 2.

- 3.3.Geophysical and lithotechnical characterization

- At  the  end  of  September  2016,  ten  temporary  seismological  stations  were  installed  in       the

- municipality of Arquata del Tronto to evaluate the site response (Laurenzano et al., 2019). Two  out

- of them have been deployed in the area under study: MZ85 station (at STB site) and MZ80 station

- (at STC site). At STA site, there are no continuous recordings from seismological stations. From

- their analyses, Laurenzano et al. (2019) inferred the site response at STB and STC through both  the

- Generalized  Inversion  Technique  (GIT)  applied  to  S-wave  amplitude  spectra  of      earthquake

- recordings and the Horizontal to Vertical Spectral Ratio of seismic noise (HVSR). The results, in

- terms of spectral amplification, evidenced that site STB presents a single-peaked response at 4.5 Hz

- (0.22 s), on the seismologic station MZ85, for the horizontal components of the ground motion,

- whereas at  site STC (seismologic station MZ80) the  response  exceeds the value  2 over  a    broad

- range of values, from 1.5 to 4.5 Hz (0.22 - 0.67 s). Moreover, many geophysical investigations were

- carried  out  during  Seismic  Microzonation  studies  (ISPRA,  2017).  At  the  site  STC,  from  the

- background  noise  recording  (with  Horizontal  to  Vertical  Spectral  Ratio  analysis),     amplified

- frequencies of about 3Hz (0.33 s) were found. At site STB, amplification peaks at 4.5 Hz (0. 22 s)

- can be definitively recognized. At site STA, investigation results captured frequencies between 2

- and 3 Hz (0.3 - 0.5 s). From the information retrieved, we decided to group the four lithofacies of

- the Laga Formation into two, according to their geophysical and mechanical properties, as shown in

- Table 2. As a result, we use 2 litho-technical units, one predominantly arenaceous (i.e. LAG4c+

- LAG4d),  the  other  mainly  pelitic  (i.e.  LAG4e+LAG4b),  in  addition  to  the  Quaternary deposits,

- belonging to a single litho-technical unit, which onlaps over all the alternations of the various

- lithofacies of the LAG4  member. In the seismic response analyses, a shallow layer of fracturing and

- alteration was also considered.

- Table 2

- Material properties used in both 2D and 3D numerical simulation models.

| Vs |  |  | Vp | Density ρ | Curves G/G0 e |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (m/s) | (m/s) | (kg/m3) | D/D0 |  |  |
|  | Alluvial deposits | 450 | 1100 | 1800 | Rollins et alii | 45 | 160 |
|  | Alteration layer | 700 | 1700 | 2000 | Linear D=1% | 70 | 245 |
|  | Pelitic association | 900 | 2250 | 2100 | Linear D=0.5% | 90 | 315 |
|  | Arenaceous (bedrock) association | 1000 | 2500 | 2200 | Linear D=0.5% | 100 | 350 |
| 293 |  |  |  |  |  |  |  |

Layers

Qμ	QK

- The material properties were obtained from previous studies (Puzzilli et al., 2019; Ciancimino et al.,

- 2019) and from investigations conducted during the data acquisition survey (investigations and

- results from Level 3 microzoning activities at Arquata del Tronto municipality can be found on

- http://www.halleyegov.it/c044006/hh/index.php,   last   access   October,   2020).   We   define   the

- characteristics of the arenaceous association from a Down Hole (DH) investigation, carried out at

- site STA. Based on other DH investigations, the parameters for the pelitic association have also

- been defined. Vs =700 m/s was assigned to the alteration layer and Vs = 450 m/s to the deposits

- based on Multichannel Analysis of Surface Waves (MASW) tests. MASW results were preferred to

- DH investigations, as the former is indicative of average properties along a transept while the  latter

- represent punctual measurements.

- 3.4. 3D digital geologic model

- Since  3D  subsoil  geological  models  represent  the  basis  for  the  execution  of  3D      numerical

- simulations,  a  digital  geological  3D  model  of  the  study area  has  been  built  by integrating the

- collected surface and subsurface data with a computer-aided process. To this aim, we adopted

- GeoModeller (Calcagno et al., 2008; Guillen et al., 2004), a commercial software which allows

- building  complex  and  implicit  3D  geological  models  directly  from  geological     observations.

- GeoModeller was recently used to build up a structural and geophysical 3D digital model of the  Po

- plain sector involved in the 2012 seismic sequence with the purpose to perform physics-based

- numerical  simulations  of  earthquake  ground  motion  (Klin  et  al.,  2019).  More  details     about

- GeoModeller can be found in Appendix B.

- The 3D geological model that was set up for Arquata del Tronto covers a square area of 2 km x 2

- km down to a depth of 1 km (Table 3) and is centred at STA site.

- Table 3

- Dimensions and spatial position of the 3D model. Coordinates are given in WGS 1984 UTM zone 33N

- reference system.

|  | Min lon (km) | Max lon (km) | Min lat (km) | Max lat (km) | Min alt (m) | Max alt (m) |
| --- | --- | --- | --- | --- | --- | --- |
|  | 359606 | 361596 | 4736026 | 4738016 | 200 | 1200 |
| 320 |  |  |  |  |  |  |

- The model was built by importing into GeoModeller the following data:

- -	a high-resolution Digital Terrain Model at a grid size of 10 m of the area (Fig. 6a)  (Tarquini

323	et al., 2007);

- -	an   excerpt   of   the	geological-geomorphological   map   for   the	third   level   Seismic

- Microzonation studies of the Macro Area 1 Arquata del Tronto and Montegallo (AP) (ISPRA,

- 2017), which reports the main geological units outcropping in the area, as well as the tectonic

- structures (Fig. 2);

- -	three geological sections (Figs. 3, 4, 5): 1-1’ crossing site STB and Camartina localities in

- almost EW direction, 3-3’ almost parallel to 1-1’ and crossing both STA and STC sites, 2-2’

- transversal to 3-3’ section and crossing both STA and STB sites.

- To constrain the geometry of the different lithofacies, we manually digitized, on the geological-

- geomorphological map and along the cross-sections, the interfaces between them as well as the fault

- traces, attributing to the latter their extension, their relationship with the geological succession   and

- their relationship with other faults. In GeoModeller, the building of the 3D model includes    several

- steps, with a progressive integration of the available data. At each step, we perform    a computation

- of the implicit surfaces of the lithofacies boundaries and check the partial result before adding   new

- elements. The final 3D geological model obtained by GeoModeller is displayed in Fig. 6b. The

- volumes corresponding to the different geophysical bodies are shown.

- Fig. 6. a) Digital Elevation Model; b) 3D model computed by GeoModeller. The model is built according to

- a cartesian reference frame, where x-axis points East and y-axis points North. The spatial coordinates of the

- x and y axes are referred to the kilometric reference system WGS 84 UTM zone 33 N. The units of

- measurement of the z-axis refer to the meters above sea level.

# 4.   Numerical modelling of seismic wave propagation

- 4.1. 2D numerical simulation

- To  confirm  the  expected  different  seismic  responses  at  different  morpho-litho-seismic        site

- conditions,  STA  and  STB  sites  have  been  studied  employing  2D  numerical  simulations   and

- compared to the 3D analyses.

- 4.1.1.   2D numerical simulation settings

- 2D numerical analyses have been performed through the commercial code LSR2D by STACEC

- (www.stacec.com) based on the finite element method. Its theoretical references are illustrated in

- Appendix C. Fig. 7a shows the section 2-2’, where the 2D equivalent linear numerical analyses

- have been accomplished. The section has been divided into 37244 triangular 3 m wide elements

355	(Fig. 7b).

356

- Fig. 7. (a) The spatial domain for 2D numerical simulations with the LSR2D code (Stacec s.r.l.) across

- section 2-2’.  Arquata del Tronto and Borgo hamlets. Legend: yellow=pelitic association; light blue=alluvial

- deposits; grey= alteration layer. The red points on the surface are the measuring points of the seismic signal.

- (b) Mesh and boundary conditions of the 2D model.

- 4.1.2.   Input motion for 2D simulation

- The input motion, given at the bottom of the model domain, is a set of seven accelerograms that are

- compatible (within the range 0.1-1.1 s) to the target acceleration spectrum provided by the Italian

- Building Code (NTC18), assuming a horizontal acceleration with a probability of exceedance of

- 10% in 50 years, referring to stiff soils (Vseq > 800 m/s; cat. A) according to the Reference Italian

- Seismic  Hazard  Map  (GdL MPS,  2004  approved  by the  Decree  of  the  Prime  Minister OPCM

- n.3519/2006.  The  seven  accelerograms  and  related  spectra  (Fig.  8)  were  selected,  for        the

- municipality of Arquata  del  Tronto,  by Luzi  et  al.  (2016) during  the  Central  Italy third     level

- Microzonation study (ISPRA, 2017). All the records are related to normal fault source   mechanism

- and reference site A*.

- Fig. 8. Time histories and response spectra of seven accelerograms used as input motion for numerical

- simulations (Luzi et al., 2016).

- 4.2. 3D numerical modelling

- The continuous development of advanced numerical methods and computational power facilities

- have made technically feasible the simulation of seismic wave propagation in 3D heterogeneous

- media and the computation of realistic seismic ground motion records at the surface in a   frequency

- range of interest in seismic design applications. As a consequence, it has arisen the interest for the

- integration of such 3D physics-based numerical simulations in seismic hazard analyses to further

- improve seismic ground motion prediction. 3D physically based numerical simulations can  provide

- a detailed and accurate characterization of the seismic ground motion, allowing the exploration of

- its spatial  heterogeneity.  Therefore, they appear  as a powerful  and efficient  tool  for an   accurate

- estimation of local seismic amplification effects that are useful in understanding the differentiated

- damages caused by strong earthquakes.

- 4.2.1.   3D response functions

- The seismic site effect is usually regarded as a time-invariant linear system, therefore we  evaluated

- the site effects over the Arquata del Tronto hill and its surroundings in terms of 3D impulse

- response functions, as described in Paolucci et al. (1999). The 3D response function consists of a

- 3×3 matrix with components hij(x,t). The time series hij(x,t) describes the ith  spatial component of

- the seismic time series that would be observed at the site x when the area is invested by a  vertically

- incident impulsive plane wave polarized in the jth  direction with unitary amplitude. We can  express

- the modified ground motion y(x,t) = [y1(x,t), y2(x,t), y3(x,t)] at the site x for any vertically incident

- seismic input u(t) = [u1(t), u2(t), u3(t)] as

- yi (x,t) = Σj hij (x,t) * uj (t)	(1)

- where * denotes convolution in time. The expression is usually evaluated in the frequency  domain,

- substituting the convolution with multiplication by the convolution theorem

- Yi (x,f) = Σj Hij (x,f) Uj (f)	(2)

- where uppercase letters denote the Fourier transforms of the corresponding time-series denoted   by

- lowercase letters.

- In  the present  work,  we  evaluated  the theoretical  response functions  hij  (x,t) on  the topographic

- surface of a given 3D structural model from the time domain numerical solutions of the visco-

- elastodynamic equation for three cases of a vertically incident plane wave: 1) shear wave  polarized

- in the East-West direction, 2) shear wave polarized in the North-South direction, 3)   compressional

- wave (Up-Down polarization).

- 4.2.2.   Spectral Elements Method

- Physics-based numerical modelling of seismic wave propagation in 3D heterogeneous  visco-elastic

- media at frequencies of engineering interest (i.e. of the order of 10 Hz) represents a demanding

- computational task. Typically, the computational cost requires the usage of parallel algorithms   and

- high-performance  computing  (HPC)  resources.  Several  accurate  and  computationally   efficient

- numerical methods for modelling the earthquake ground motion in 3D heterogeneous media has

- been developed so far (e.g. Moczo et al., 2014). The method we adopted hereinafter is called the

- Spectral Element Method (SEM), that was introduced in the study of acoustic and elastic waves   by

- Seriani and Priolo (1994). SEM is a high order numerical method that combines the geometrical

- flexibility of Finite Elements Methods (FEM) with the accuracy of the spectral methods. Like FEM,

- SEM  is  based  on  the solution  of the variational  formulation  of the equation  of motion  in   non-

- overlapping spatial subdomains (elements), in which we subdivide the investigated volume.  Unlike

- FEM, SEM represents the wavefield in each element in terms of spectral polynomial bases of a high

- order, with a significant gain of accuracy. SEM has proved to be effective in modelling   earthquake

- ground motion (e.g. Komatitsch and Vilotte, 1998; Peter et al., 2011), and thanks to the geometrical

- flexibility  it  resulted  particularly  suitable  for  cases  where  an  irregular  topographic  surface  is

- involved (e.g. Lee et al., 2009). To apply the SEM method to the case study discussed in this  work,

- we	adopted	the	open-source	software	SPECFEM3D	Cartesian

- (https://geodynamics.org/cig/software/specfem3d/, last  access 28th  June 2019), which consists     of

- implementing the SEM based on the discretization of the spatial domain in hexahedral elements and

- the  wavefield  expansion  in  Legendre  polynomials  (Komatitsch  and  Tromp,  1999).  The   code

- accounts  for the intrinsic attenuation  properties  of the viscoelastic medium  by a series  of    Zener

- standard linear solids (Komatitsch and Tromp, 2002a) and avoids the boundary reflection   artefacts

- thanks to the usage of Perfectly Matched Layers (Xie et al., 2014).

- SPECFEM3D is suitable to run on HPC machines, where we can shorten its execution time by

- distributing the computational tasks over a high number of processors. The computations related  to

- the present work were performed using the HPC resources available at CINECA through the Italian

- SuperComputing Resource Allocation service - ISCRA (http://www.hpc.cineca.it/services/iscra).

- 4.2.3.   3D numerical simulation settings

- The  most   demanding  step   in   the  preparation   of  a  numerical   simulation   of  seismic  waves

- propagation with SPECFEM3D is the definition of the computational spatial domain, that   consists

- in an unstructured hexahedral mesh according to the geometry of the 3D geomodel. The   numerical

- modelling in the present  work is based  on a mesh that was  generated  through the    SPECFEM3D

- internal meshing machine with the following specifications:

- -	horizontal base: a square area at -500 m a.s.l. with 3 km long sides aligned with the  cardinal

- points and centred at STA site;

- -	top surface geometry: 10 m-resolution TINITALY DEM (Tarquini et al., 2007);

- -	element size: ranging from 25 m at the topographic surface to 50 m at the flat bottom

- surface with doubling layer at sea level (Fig. 9).

- Given the element size at the topographic surface, we neglected the presence of the thin deposit

- layer except for the limited area around site STB (Fig. 9), where a thicker layer of deposits was

- detected (ISPRA, 2017).

- Fig. 9. SPECFEM3D mesh for the 3D geophysical and lithotechnical model centred at the study area. The

- spatial coordinates of the x and y axes are referred to the kilometric reference system WGS 84 UTM zone 33

- N. The units of measurement of the z-axis refer to the meters above sea level.

- The resulting mesh consists of 518,400 elements: each of them was characterized by the mechanical

- properties of the structural unit occupying its position in the 3D geologic model. Considering the

- values of the mechanical properties (Table 2), the dimensions of the elements imply a time step as

- short as 0.0005 s to satisfy the stability criterion of the time marching step implemented in the

- SPECFEM3D solver and that accurate solutions of seismic waves up to the frequency of 10 Hz  can

- be obtained. On the other hand, the finiteness of the overall size of the considered spatial domain

- undermines the meaningfulness of the modelling results for frequencies below 1 Hz.

- The polarized vertically incident plane waves, that were required for the evaluation of the  response

- functions, were generated with a horizontal carpet of point sources at -400m a.s.l, with an explosive

- mechanism for the vertically polarized plane wave and West-East or North-South oriented double

- couple  mechanism  for   the  two   independent  cases  of   horizontally  polarized  plane       waves,

- respectively.

- The SPECFEM3D solver was configured to output 8 seconds long three-component time series

- evaluated in a dense regular grid of points on the topographic surface, on three cross-sections   2-2’,

- 2-2’bis and 3-3’ (Fig. 2) and in three points STA, STB and STC. STB and STC, corresponding to

- the location of the temporary stations MZ85 and MZ80, were used to compare the numerical results

- with site observations in the present paper.

- The  computational  cost  of  each  run  amounted  around  900  CPU  hours  distributed  among 225

- processing units of the Marconi supercomputer at CINECA.

# 5.   Results and discussion

- In the following, we first evidence the variability of the site effects in the studied area as they result

- from 3D numerical simulations by plotting the horizontal transfer functions along some lines across

- the area under investigation (subsection 5.1). Next, we attempt validation of the 3D simulations at

- the two sites STA and STB, using horizontal to vertical spectral ratios from earthquake   recordings

- described  in  Laurenzano  et  al.  (2019), (subsection 5.2).	Finally,  we  point  out  the differences

- between the 3D and 2D simulations in terms of the response spectra for specific seismic input at

- sites STA and STB and in terms of amplification functions and Housner’s amplification factors

- evaluated along the line 2-2’ (subsections 5.3 and 5.4).

- 5.1. 3D transfer functions

- In Fig. 10, we plot the Fourier amplitude spectra of the NS and EW component of the numerical

- response obtained from the 3D model for a vertically incident shear plane wave polarized in the NS

- and EW direction, respectively, along the lines 2-2’, 2-2’bis and 3-3’ (see map in Fig. 2). In the

- bottom panels, we show a vertical cross-section along the same lines to correlate the changes in  the

- spectra with the topographic profile and structural features.

- Sections 2-2’ and 2-2’bis are almost N-S oriented and therefore the NS and EW components

- correspond  approximately  to  the  response  of  in-plane  (P-SV)  and  out-of-plane  (SH)  motions,

- respectively, whereas section 3-3’ runs nearly in the E-W direction and the interpretation of the

- components in terms of in-plane and out-of-plane motion is exchanged there.

- Fig. 10 points out a prominent spatial variability in the frequency-dependent site effects predicted

- with   the  numerical   simulation.   The  largest   amplification   values   are  observed   on   the N-S

- component, which is perpendicular to the main axis of the relief (the ridge is approximately E-W

- oriented). For both the components, we can observe that the largest amplification is concentrated  in

- correspondence  with  the  top  of  the  relief  with  multiple  peaks  corresponding  to  the  modes of

- vibration of the hill. Moving away from the top, we observe an alternation of amplification and  de-

- amplification along the slope due to constructive and destructive interferences of the seismic  waves

- inside the hill structure, a typical topographic effect already evidenced by Komatitsch and Vilotte

- (1998). The base of the relief is characterized by low values and, in some cases, de-amplification

- effects occur.

- Fig. 10. Transfer functions obtained from 3D simulations on three studied sections. (a) cross-section 2-2’ bis;

- (b) cross-section 3-3’; (c) cross-section 2-2’.

- 5.2. H/V spectral ratios from earthquake recordings

- A large number of seismic recordings was collected at sites STB and STC during the 2018   seismic

- sequence  (Laurenzano,  2019).  However,  because  of  survey  logistics  (the  same  instrument was

- installed at the two sites during different time intervals), no simultaneous recordings at the two

- stations  exist,  and  a  validation  attempt  for  3D  numerical  simulations  in  terms  of  the relative

- amplification of the same component between sites STB and STC was not feasible. We therefore

- resorted to the description of the site effect in terms of the horizontal-to-vertical (H/V) spectral ratio

- of earthquake recordings (Lermo and Chavez-Garcia, 1993). We simulated a set of earthquake

- recordings  at  stations  STB  and  STC  by  means  of  convolution  of  the  numerical  3D  response

- functions at STB and STC with a set of almost two hundred seismic inputs. As seismic inputs, we

- considered the recordings collected during the same survey by Laurenzano et al. (2019) at the

- station MZ75 located on geological bedrock at about 12 km far from hamlet Uscerno.

- In Fig. 11, we display the mean H/V spectral ratios for experimental and simulated data (black   and

- blue lines respectively). The shadow area represents the standard error and the two dashed lines

- represent the mean for the two horizontal components considered separately.

- For  site  STC,  the  curves  obtained  from  the  simulated  ground  motion  are  characterized  by an

- alternation of peaks (at about 2, 3 5 and 9 Hz) and notches between them, with a low-moderate level

- of amplification (below 3). The curves obtained from the observed ground motion exhibit a  similar

- pattern but with higher values for frequencies below about 4.5 Hz and lower values for higher

- frequencies. The underestimation of the H/V in the low-frequency part of the spectrum may be  due

- to the limited extension of the spatial domain that we considered in the simulations. In fact, the

- Arquata hill is part of a larger structure, which is truncated and smoothed out in our 3D model   and

- whose contribution to the site response results neglected.  On the other hand, the polarization effects

- evidenced by the difference between the separate horizontal components are rather consistent in the

- simulated and observed data.

- For site STB, located in the valley, the simulated data predicts a mean peak value of about 2 at

- about 5 Hz instead of the value 3 at about 4.5 Hz given by observed data, but these differences are

- in the order of magnitude of the standard error.

- Even though these comparisons do not provide a strict validation for the adopted 3D model, they

- suggest a substantial correctness in the adopted parametrization.

- Fig. 11. H/V spectral ratios from earthquake recordings calculated by 3D simulations and experimentally by

- earthquake recordings by Laurenzano et al. (2019) at two sites of Arquata del Tronto municipality: (a)Site

- STC – MZ80 (the Castle over the hill); (b) Site STB – MZ85 (Borgo hamlet, on a sediment-filled basin).

- Dash-dotted and dashed lines refer to EW and NS, respectively

- 5.3. Site-specific response spectra

- The response spectra for 5% damping have been estimated from 2D and 3D simulations, given a

- seismic input compatible with the Uniform Hazard Spectrum, calculated for a 10% exceedance

- probability level in 50  years  (return  period of 475  years),  as defined by the  Italian  seismic  code

- (NTC18). See section 4.1.2 for all the details about the characteristics of the seismic input. In

- particular, we obtained the 3D response spectra by the convolution of the seismic input with the 3D

- model response functions (eq. 1).

- Fig.  12  shows  the  Site-Specific  Response  Spectra  obtained  from  3D  simulations  at  the  three

- selected sites (blue lines). At sites STA and STB, they are compared with those obtained by 2D

- simulations (green lines). For sites STB and STC we also plot (red dotted lines) the Site-Specific

- Response Spectra obtained from the convolution of the seismic input with the experimental site

- amplification curve calculated for a rock reference site by the site-response analysis of  Laurenzano

- et al. (2019). The spectra of the Italian building code NTC18 for the ground types and topography

- category appropriate for the three sites (i.e. A and T3 category for sites STA and STC, STB and  T1

- for site STB) are also reported (dashed orange lines). We can observe that, for all the cases, the

- NTC18 spectra underestimate the amplification recorded in the seismological stations, and therefore

- underestimate the numerical response spectra, for periods higher than 1s, for sites STA and STC,

- and higher than 0.5s for site STB. The downfall of the response spectra can be probably attributed

- to the limited dimension of the spatial domain considered in, both 2D and 3D simulations. On the

- other hand, in the period band 0.1 - 1 s, the NTC18 curves underestimate both the numerical and the

- experimental spectra up to a factor of 2. Differences between 2D and 3D curves are not that relevant

- in shape, but the mean 3D response spectra are always greater than the 2D ones.

- At site STA (Fig. 12a), the response spectra from 2D and 3D simulations exhibit similar  behaviour,

- with two peaks around 0.15 s and 0.25 s, although 3D predicts higher values than 2D.	These

- amplifications are almost uniform over the upper part of the hill for the 2D simulations, as can be

- seen from the little dispersion of the response spectra in light green. On the contrary, the  dispersion

- of the response spectra amplifications is quite large from the 3D simulations, with a range of  about

- 0.4 g along the spectral ordinate.

- The simulated response spectra at site STB fit quite well with the experimental one (Fig. 12b): the

- predominant peak at 0.2 - 0.25 s is predicted by both 2D and 3D simulation, thought slightly shifted

- toward higher periods.

- At site STC, (Fig. 12c) the comparison between 3D and experimental spectra shows differences,

- especially regarding the shape of the curves. Amplitudes instead are quite similar, approximately

- for all periods but especially below 0.3 s. This site appears to be the most amplified among the three

- analysed, with the maximum values of about 1.5 g, compared to NTC18 curve for which a   spectral

- acceleration of 0.7 g is expected on the plateau.

- Fig. 12. Acceleration response spectra calculated by 2D and 3D simulations at the three sites of Arquata del

- Tronto municipality: (a) Site STA (Arquata del Tronto Old Town, on the ridge); (b) Site STB – MZ85

- (Borgo hamlet, on a sediment-filled basin); (c) Site STC – MZ80 (the Castle over the hill), red dash-dotted

- and dashed lines refer to GIT EW and NS component, respectively.

- 5.4. Amplification functions and Housner’s amplification factor

- To have a better insight on the differences between the 2D and 3D simulations we compared the

- two in terms of the amplification function and the Housner’s amplification factor along the line 2-2’

- which includes the sites STA and STB. We define the amplification function as the ratio between

- the response spectra of the simulated ground motion and the response spectra of the corresponding 600      seismic input. On the other hand, we define Housner’s amplification factors as the ratio of the       601    Housner’s Spectral  Intensity (SI) of the simulated ground motion and the corresponding seismic    602       input:

J

=

n1

nz

J

603	nnn		nz n

PSVoutnn PSVinnn

(3)

604 where  PSVout  is  the  output  pseudo-velocity spectrum  at  a  location;  PSVin  is the  input  pseudo- 605      velocity spectrum at the same location of the PSVout; T1  and T2  represent the extremes of two-      606  period  ranges  of  calculation.  We  considered  two-period  ranges:  a  “short”  period  range  with   607       extremes T1=0.1 s and T2= 0.5 s and a “long” period range with extremes T1=0.5 s and T2=1.0 s.

608     In Fig.13 (I and II panel) we compare the amplification functions (the ratio between output and      609   input acceleration spectral responses) obtained from 2D and 3D simulations along section 2-2’ for   610    the  SH  component  of  ground  motion.  The  2D  model  predicts  the  largest  amplification  in    611   correspondence  of the  sediment  valley (where site STB is  located) with  the amplification  level   612 larger than 2 between 4 and 6 Hz (T=0.17 - 0.25 s) whereas at site STA the predicted amplification  613   is lower and involves a frequency range restricted to between 2.5 and 3.5 Hz (T=0.29 - 0.4 s). On    614    the contrary, the 3D model predicts a lower amplification at site STB while at site STA and in its    615  surroundings corresponding to the topographic relief it predicts large amplification values over the   616   whole considered frequency range. We can conclude that in comparison to the 3D simulations the   617    2D simulations underestimate the topographic effects and overestimates the valley effects. These    618   results are summarized in the curve of the Housner’s amplification factors calculated from 2D and   619  3D  simulations  along  the  section  (Fig.13,  III  panel)  in  two-period  ranges:  FA1  =  [0.1-0.5]  s 620       (continuous line), FA2  = [0.5-1] s (dashed line).

621                It appears that in the studied case, the amplification levels evaluated with the 3D simulation are

622       consistent with the observed damage distribution while the 2D simulation contradicts it.

623  Fig. 14 illustrates the values of the Housner amplification factors evaluated in the two-period ranges 624       from 3D simulations over the whole studied area. The short-period band (FA1) presents local       625 amplification patches as the one on the top of the hill (site STC), the Arquata del Tronto centre (site 626 STA), the surrounding ridges and the Borgo alluvial basin. On the other hand, the long-period band  627   (FA2) is  characterized  by an  amplification  next  to  2  at STA and  its neighbourhood,  but  it  is   628       negligible in the basin where Borgo (site STB) is located.

629

630		Fig. 13.  Amplification functions calculated by 2D (a) and 3D (b) simulations along the cross-section 2-2’. 631		Housner’s amplification factors are calculated (c) along section 2-2’ by both 2D and 3D simulations. Blue 632	curves: 3D model; green curves: 2D model. Full line: 0.1- 0.5 s; dashed line: 0.5 - 1 s. Cross-section to which 633			the Housner's amplification factors refer is represented in (d).

634        Fig. 14. Housner’s amplification factors FAH over the study case area, calculated in the period range: 0.1-0.5

635	s (a) and 0.5-1 s (b) from 3D simulations.

636

637  To visualize the dependence of the response of the 3D model to the polarization of the solicitation,   638    we plot in Fig. 15 the peak values of the two horizontal components of the time-domain response    639       function corresponding to a soliciting impulse of the same direction.

640 Thanks to the nearly East-West orientation of the Arquata del Tronto ridge, we don’t need to rotate  641  the NS and EW component of the response function to investigate the response in the ridge-parallel  642    and  ridge-perpendicular  components.  From  Fig.  15  it  appears  quite  clearly  how  the  ridge-    643     perpendicular component (the NS one) exhibits the largest peak values along the ridgetop. This      644  behaviour could be reasonably associated with the amplification of the vibration of the ridge in the   645 component perpendicular to its elongation observed by He et al. (2020) and Pischiutta et al. (2010).  646     On the other hand,  we  can observe that the peak values  of the other component (the EW one)      647      present local maxima on the secondary NS oriented crests that descend from the main ridge in      648   correspondence of the outcrop of the arenaceous lithofacies. The points on the ridgetop located on   649  arenaceous   lithofacies   are  therefore  subjected   to   the  amplification  in   both   the  horizontal   650       components, which could explain the heavy damaging observed there.

651

652  Fig. 15.  The distribution of the peak values of the time-domain response function horizontal components (a) 653  and horizontal components (b) evidences the dependence of the response of the 3D model to the polarization 654	of the upcoming S-wave.

655    An interesting aspect of  the distribution of the  ground  motion throughout the surface  of the 3D    656     model  emerged  by  representing  the  resultant  vector  of  the  horizontal  components  of  the      657   instantaneous  acceleration  vectors.  Fig.  16  shows  a  snapshot  of  the  horizontal  accelerations   658   obtained with the convolution of the 3D response functions with the signal input corresponding to   659  the  recording  of  the  Mw=5.4  foreshock  of  the  day  30th  October  2016  in  the  station  T1212   660  (Avendita) at an epicentral distance of about 15 km.  For each point, the direction of the resultant is  661      represented as a vector. The snapshot is extracted from the arrival of the S waves train and the      662 vector  length  is  proportional  to  the  acceleration  amplitude.  It  can  be  observed  that  along  the 663  secondary ridge (about NS oriented) on which Arquata del Tronto centre is located, the vectors are   664 not  aligned  along with  preferential  directions  but  describe  a  rotary motion.  The  fact  that  two  665  neighbouring points  have two  different  vector directions  creates  a twisting effect  in  the ground, 666   which likely could be transferred to the structures above. This torsional component of the ground    667     motion, due to the ground motion incoherence of the asymmetric ridge (Anagnostopoulos et al.,     668     2015), can generate an accidental eccentricity that adversely affects most of the building owing      669       generalized severe damage or collapse.

670

671        Fig. 16.  Distribution of the instantaneous acceleration vectors over the surface of the 3D model on the ridge

672	where Arquata del Tronto hamlet is located.

673

# 674	6.   Conclusions

675 The  complete  disruption  suffered  by  Arquata  del  Tronto  town,  Central  Italy,  set  on  a  rocky  676   asymmetric ridge  made  up of the  Laga  Formation and, on the  contrary,  lighter  damage pattern   677   observed at Borgo hamlet, set on soft to stiff alluvial valley deeper than 30 m, induced the writing   678   authors to investigate the complex seismic behaviour of the Arquata del Tronto rocky ridge by not   679     only  2D  but  also  3D  numerical  simulations  of  physics-based  modelling  of  seismic  wave     680   propagation. Comparing the numerical simulation results in terms of acceleration response spectra   681       and amplification functions, the following conclusions can be pointed out:

682    - At Borgo site, 2D simulations overestimated the level of seismic amplification compared to 3D    683      simulations  even  though  at  the  same  resonant  frequencies.  The  differences  in  predicted       684    amplification amplitudes can be reasonably referred to the simplification  of the  geometry in 2D    685       models and the trapping of the energy in a smaller area rather than the 3D actual volume;

686   - At Arquata del Tronto old town, 2D simulations predicted amplifications that are lower than those 687      at Borgo site. Instead, the 3D simulations predicted large amplification over the ridgetop and a       688        complex    pattern of amplification and de-amplification along the flanks of the asymmetric hill. The

689 amplification along the ridgetop is especially effective in the NS component which is perpendicular  690 to the elongation of the ridge. Nonetheless, amplifications along the secondary oriented crests have  691     been observed (EW direction) where arenaceous lithofacies outcrop. The points on the ridgetop     692  located on arenaceous lithofacies are therefore subjected to the amplification in both the horizontal   693       components, which could explain the heavy damaging observed there,

694          - At the Castle site, the 3D acceleration response spectra correctly reproduced the main features of

695       the strong motion event recorded there on 30 Oct 2016.

696  - The study of kinetics at Arquata del Tronto ridge highlighted that twisting particle movements can 697  be traced in the horizontal plane, that likely caused additional solicitations responsible for the large  698       disruption suffered by Arquata del Tronto ridge site.

699    The present study demonstrated the need for 3D simulations instead of 2D ones, commonly used    700 even  when  the  plane  symmetry conditions  related  to  both  the  topography and  the  geology are 701 broken. Additionally, 3D simulation results are influenced by the accurateness of the geological and 702     geophysical 3D digital model reconstruction which allowed us to predict a plausible map of the     703   ground motion amplification effects in a restricted area around Arquata del Tronto hamlet, for the    704       frequency band 1-10 Hz.

705   The map results are compliant with the observed damage distribution and evidence the role of the    706  combination of topographic and stratigraphic effects for the seismic hazard in morphologically and   707  geologically complex areas, like those in the Central Apennines. For this reason, the authors suggest 708 using 3D simulations at third level microzoning activities, wherever urban areas are set on or nearby 709   complex rocky relieves especially when a weathered rocky mass level is detected. To this end, an    710 adequate amount of geological and geophysical data must be collected to reconstruct representative  711       3D models of the ridges.

712

713       Author contribution

714    The ISPRA working group (C.M., F.C., M.A.) developed the update of the geological settings at Arquata del Tronto    715   presented  in Section 3.1; INOGS  working Group  (J.B., G.L., P.K.) together  with I.P. undertook the 3D  models and   716   simulations; UdA working group (G.V., I.P.) accomplished the 2D model and simulations. All the authors contributed   717      to writing the paper and approved the results drawn. The coordination of the research group has been addressed by G.V.

718

- Acknowledgements

- We would like to thank Luca Maria Puzzilli, Mauro Roma, Vittorio Chiessi and Fernando Ferri for their contribution in

- the definition of the geological and geophysical model of the study area.

- We acknowledge the CINECA award under the ISCRA initiative, for the availability of high performance computing

- resources and support.

- The  authors  are  grateful  to  scientists  from  the  Microzoning  Center  (CentroMS)  for  the  fruitful  discussions  and

- suggestions.

- Signal  processing  and  figures  have  been  done  with  Matlab  2018,  9.5.0.1067069  (R2018b)  Update  4,      Natick,

- Massachusetts: The MathWorks Inc.

- We thank the Computational Infrastructure for Geodynamics (http://geodynamics.org) which is funded by the  National

- Science Foundation under awards EAR-0949446 and EAR-1550901, for the availability of SPECFEM3D code.

- This study has been developed within the framework of the Research agreement n. 47058 on 02/07/2019 between the

- OGS National Institute and UdA University.

- Finally, the authors are grateful to the anonymous reviewers and the endling editor, for the valuable comments that

- highly improved the paper.

- Funding sources

- The research reported in this work was supported by INOGS and CINECA under HPC-TRES program award number

737       2017-22 (J. B.).

738       I.P.  received  a  Master  thesis  award  protocol  n.  154/2019  [UOR:  D05  -  Classif.  III/2]  from  the  Department   of

739       Engineering and Geology (INGEO) - “G. d’Annunzio” University for this research work.

740

741       ORCID

742       Ilaria Primofiore https://orcid.org/0000-0002-5688-8067

743       Julie Baron https://orcid.org/0000-0002-3215-6996

744       Peter Klin https://orcid.org/0000-0002-5373-4484

745       Giovanna Laurenzano https://orcid.org/0000-0002-2618-3007

746	Marco Amanti https://orcid.org/0000-0002-2219-3558 747	Giovanna Vessia https://orcid.org/0000-0003-1733-7112 748

749       References

750   Amanti, M., Muraro, C., Roma, M., Chiessi, V., Puzzilli, L., Catalano, S., Romagnoli, G., Tortorici, G., Cavuoto, G.,    751	Albarello, D., Fantozzi, P.L., Paolucci, E., Pieruccini, P., Caprari, P., Mirabella, F., Della Seta, M., Esposito, 752	C., Di Curzio, D., Francescone, M., Pizzi, A., Macerola, L., Nocentini, M., Tallini, M., 2020. Geological and 753	geotechnical models definition for 3rd level seismic microzonation studies in Central Italy. Bull.  Earthq. Eng. 754	https://doi.org/10.1007/s10518-020-00843-x

755       Anagnostopoulos, S.A., Kyrkos, M.T., Stathopoulos, K., 2015. Earthquake induced torsion in buildings: Critical review

756	and state of the art. Earth. and Struct., 8(2): 305-377. https://doi: 10.12989/eas.2015.8.2.305

757    Barani, S., Massa, M., Lovati, S., Spallarossa, D., 2014. Effects of surface topography on ground shaking prediction:    758	Implications  for  seismic  hazard  analysis  and  recommendations  for  seismic  design.   Geophysical  Journal 759	International. 197. https://doi.org/10.1093/gji/ggu095

760   Burjanek, J., Faeh, D., Pischiutta, M., Rovelli, A., Calderoni, G., Bard, P.-Y., NERA-JRA1 working group, 2014. Site   761	effects at sites with pro-nounced topography: overview & recommendations, Research report for EU project 762	NERA, 64 pp.

763      Calcagno, P., Chilès, J.P., Courrioux, G., Guillen,  A., 2008. Geological modelling from field data and geological       764	knowledge. Part I. Modelling method coupling 3D potential-field interpolation and geological rules. Phys. 765	Earth Planet. Inter. https://doi.org/10.1016/j.pepi.2008.06.013

766           Çelebi, M., 1991. Topographical and geological amplification: case studies and engineering implications. Struct. Saf.

767	https://doi.org/10.1016/0167-4730(91)90015-2

768       Centamore, E., Adamoli, L., Berti, D., Bigi G., Bigi, S., Casnedi, R., Cantalamessa, G., Fumanti, F., Morelli, C.,        769	Micarelli, A., Ridolfi, M., Salvucci, R., 1991. Carta geologica dei bacini della Laga e del Cellino e dei rilievi 770	carbonatici  circostanti  (Marche   meridionali,   Lazio   nord-orientale,  Abruzzo   settentrionale). Studi  Geol. 771	Camerti, Vol. Spec. 1991/2.

772  Chiles, J.P., Aug, C., Guillen, A., Lees, T., 2007. Modelling the geometry of geological units and its uncertainty in 3D   773	from  structural  data  -  The  potential-field  method,  in:  Australasian  Institute  of  Mining    and  Metallurgy 774	Publication Series.

775   Ciancimino, A., Lanzo, G., Alleanza, G.A., Amoroso, S., Bardotti, R., Biondi, G., Cascone, E., Castelli, F., Giulio, A.   776	Di, D’Onofrio, A., Foti, S., Lentini, V., Madiai, C., Vessia, G., 2019. Dynamic characterization of fine-grained 777	soils in central Italy by laboratory testing. Bull. Earthq. Eng. https://doi.org/10.1007/s10518-019-00611-6

778       Davis, L.L., West, L.R., 1973. Observed effects of topography on ground motion. Bull. Seismol. Soc. Am., 63(1),  283-

779	298.

780                Del Gaudio, V., Wasowski, J., 2011. Advances and problems in understanding the seismic response of potentially

781	unstable slopes. Eng. Geol. 122, 73–83. https://doi.org/10.1016/j.enggeo.2010.09.007

782   Del Gaudio, V., Luo, Y., Wang, Y., Wasowski, J., 2018. Using ambient noise to characterise seismic slope response:    783	The       case       of       Qiaozhuang       peri-urban       hillslopes       (Sichuan,       China).          Eng.       Geol. 784	https://doi.org/10.1016/j.enggeo.2018.10.008

785	Del Gaudio, V., Zhao, B., Luo, Y., Wang, Y., Wasowski, J., 2019. Seismic response of steep slopes inferred from

786		ambient noise and accelerometer recordings: the case of Dadu River valley, China. Eng. Geol. 259, 105197. 787	Ding, Y., Wang, G., Yang, F., 2020. Parametric investigation on the effect of near-surface soil properties on the 788		topographic amplification of ground motions. Eng. Geol. 273. https://doi.org/10.1016/j.enggeo.2020.105687 789	DISS Working Group, 2018. Database of Individual Seismogenic Sources (DISS), Version 3.2.1: A compilation of 790		potential sources for earthquakes larger than M 5.5 in Italy and surrounding   areas. http://diss.rm.ingv.it/diss/, 791		Istituto Nazionale di Geofisica e Vulcanologia, https://doi.org/10.6092/INGV.IT-DISS3.2.1

792	Galli P., Peronace E., Tertulliani A., 2016. Rapporto sugli effetti macrosismici del terremoto del 24 Agosto 2016 di 793		Amatrice in scala MCS. Roma, rapporto congiunto DPC, CNR-IGAG, INGV, 15 pp. 10.5281/zenodo.161323 794	Géli, L., Bard, P.-Y., Jullien, B., 1988. The effect of topography on earthquake ground motion: a review and new 795		results, Bull. Seismol. Soc. Am., 78, 42–63.

796     Giallini, S., Pizzi, A., Pagliaroli, A., Moscatelli, M., Vignaroli, G.,  Sirianni, P., Mancini, M., Laurenzano, G, 2020.     797	Evaluation of complex site effects through experimental methods and numerical modelling: the case history of 798	Arquata del Tronto, central Italy. Eng. Geol.

799	Gischig,  V.S.,  Eberhardt,  E.,  Moore,  J.R.,  Hungr,  O.,  2015.  On  the  seismic  response  of  deep-seated rock slope 800		instabilities - Insights from numerical modeling. Eng. Geol. 193. https://doi.org/10.1016/j.enggeo.2015.04.003 801	Graziani, L., del Mese, S., Tertulliani, A., Arcoraci, L., Maramai, A., Rossi, A., 2019. Investigation on damage 802		progression during the 2016–2017 seismic sequence in Central Italy using the European Macroseismic Scale 803		(EMS-98). Bull. Earthq. Eng. https://doi.org/10.1007/s10518-019-00645-w

- Gruppo di Lavoro MS, 2008. Indirizzi e Criteri per la Microzonazione Sismica. Dipartimento della Protezione Civile  e

- Conferenza delle Regioni e Province autonome. 3 vol. e 1 DVD.

- Guillen, A., Lane, R., Courrioux, G., Lees, T., Calcagno, P., Mclnerney, P., 2004. Constrained Gravity 3D Litho-

- Inversion Applied to Broken Hill. ASEG Ext. Abstr. https://doi.org/10.1071/aseg2004ab057

- He J., Qi S., Wang Y., Saroglou C., 2020. Seismic response of the Lengzhuguan slope caused by topographic and

- geological	effects,	Eng.	Geol.,	Volume	265,	105431,	ISSN	0013-7952,

810	https://doi.org/10.1016/j.enggeo.2019.105431

811  ISPRA, 2017. Relazione Illustrativa dello Studio di Microzonazione Sismica propedeutico al Livello 3 della Macroarea  812	1 Arquata del Tronto e Montegallo (AP) e Allegati - Rapporto inedito per il Dipartimento della Protezione 813	Civile,	14	novembre	2017,	Roma.	46	Carte	allegate.

814	http://www.halleyegov.it/c044006/zf/index.php/trasparenza/index/index/categoria/210

815	Kakhki, M.K., Peters, F.C., Mansur, W.J., SadidKhoii, A., Rezaei, S., 2020. Deciphering site response directivity in

816	landslide-prone slopes from ambient noise spectral analysis. Eng. Geol., 269, 105542

817  Klin, P., Laurenzano, G., Adelaide Romano, M., Priolo, E., Martelli, L., 2019. ER3D: A structural and geophysical 3-D 818	model of central Emilia-Romagna (northern  Italy)  for     numerical simulation of earthquake  ground motion. 819	Solid Earth. https://doi.org/10.5194/se-10-931-2019

820	Komatitsch,  D.,  Tromp,  J., 1999. Introduction to  the  spectral  element  method  for  three-dimensional  seismic wave

821	propagation. Geophys. J. Int. https://doi.org/10.1046/j.1365-246X.1999.00967.x

822	Komatitsch, D., Tromp, J., 2002a. Spectral-element simulations of global seismic wave propagation - I. Validation.

823	Geophys. J. Int. https://doi.org/10.1046/j.1365-246X.2002.01653.x

- Komatitsch, D., Vilotte, J.P., 1998. The spectral element method: An efficient tool to simulate the seismic response   of

- 2D and 3D geological structures. Bull. Seismol. Soc. Am., 88(2), 368-392.

- Kuhlemeyer, R.L. and Lysmer, J., 1973. Finite Element Method Accuracy for Wave Propagation Problems. Journal   of

- the Soil Dynamics Division, 99, 421-427

- Lajaunie, C., Courrioux, G., Manuel, L., 1997. Foliation fields and 3D cartography in geology: Principles of a   method

- based on potential interpolation. Math. Geol. https://doi.org/10.1007/bf02775087

- Lanzo, G., Tommasi, P., Ausilio, E., Aversa, S., Bozzoni, F., Cairo, R., d’Onofrio, A., Durante, M.G., Foti, S., Giallini, 831	S., Mucciacciaro, M., Pagliaroli, A., Sica, S., Silvestri, F., Vessia, G., Zimmaro, P., 2019. Reconnaissance of 832	 geotechnical aspects of the 2016 Central Italy earthquakes. Bull. Earthq. Eng.  https://doi.org/10.1007/s10518- 833		018-0350-8

834	Laurenzano, G., Barnaba, C., Romano, M.A., Priolo, E., Bertoni, M., Bragato, P.L., Comelli, P., Dreossi, I., Garbin, M., 835		2019. The Central Italy 2016–2017 seismic sequence: site response analysis based on seismological data in the 836		Arquata del Tronto–Montegallo municipalities. Bull. Earthq. Eng. https://doi.org/10.1007/s10518-018-0355-3 837	Lee, S.J., Komatitsch, D., Huang, B.S., Tromp, J., 2009. Effects of topography on seismic-wave propagation: An 838		example from Northern Taiwan. Bull. Seismol. Soc. Am. https://doi.org/10.1785/0120080020

839	Lovati, S., Bakavoli, M.K.H., Massa, M., Ferretti, G., Pacor, F., Paolucci, R., Haghshenas, E., Kamalian, S., 2011. 840		Estimation of topographical effects at Narni ridge (Central Italy): comparisons between experimental results 841		and numerical modelling, Bulletin of Earthquake Engineering, 9, 1987-2005, doi: 10.1007/s10518-011-9315-x 842	Luo, Y., Fan, X., Huang, R., Wang, Y., Yunus, A.P., Havenith, H.B., 2020. Topographic and near-surface stratigraphic 843		amplification  of  the  seismic  response  of  a  mountain  slope  revealed  by  field  monitoring  and  numerical 844		simulations, Eng Geol, 271, 105607

845       Luzi,  L., Pacor, F., Lanzano, G., Felicetta,  C., Puglia, R., D’Amico, M., 2019. 2016–2017 Central Italy seismic        846	sequence:  strong-motion data  analysis and  design earthquake  selection for  seismic microzonation purposes. 847	Bull. Earthq. Eng., 1-19.

848       Massa, M., Barani, S., Lovati, S., 2014. Overview of topographic effects based on experimental observations: Meaning,

849	causes and possible interpretations. Geophys. J. Int. https://doi.org/10.1093/gji/ggt341

850   Maufroy, E., Cruz-Atienza, V.M., Cotton, F., Gaffet, S., 2015. Frequency-scaled curvature as a proxy for topographic   851	site-effect       amplification       and       ground-motion       variability.       Bull.       Seismol.        Soc.       Am. 852	https://doi.org/10.1785/0120140089

853   Maufroy, E., Lacroix, P., Chaljub, E., Sira, C., Grelle, G., Bonito, L., Causse, M., Cruz-Atienza, V.M., Hollender, F.,    854	Cotton,  F.,  Bard,  P.-Y.,  2018.  Towards  rapid  prediction  of  topographic  amplification  at     small  scales: 855	contribution of the FSC proxy and pleiades terrain models for the 2016 Amatrice earthquake (Italy,   MW 6.0). 856	16th Eur. Conf. Earthq. Eng.

857            McInerney, P., Guillen, A., 2005. Building 3D geological models directly from the data? A new approach applied to

858	Broken Hill, Australia. US Geological Survey Open-File Report, 1428, 119-130.

859       Milli,  S.,  Cannata,  D.,  Marini,  M.,  Moscatelli,  M.,  2013.  Facies and  geometries of  Lower  Messinian  Laga Basin

860	turbidite deposits (central Apennines, Italy). J. Mediterr. Earth Sci, Special Issue, 179, 225.

861  Moczo, P., Kristek, J., Gális, M., 2014. The Finite-Difference Modelling of Earthquake Motions: Waves and Ruptures,  862	Finite-Difference         Modelling         of         Earthquake         Motions:         Waves          and         Ruptures. 863	https://doi.org/10.1017/CBO9781107415324.004

864   NTC18,  Norme  Tecniche  per  le  Costruzioni,  2018.  Approved  with  Ministerial  Decree  17  gennaio  2018.  Gazzetta

865	Ufficiale 42, 20‐02‐2018, Ordinary Suppl.

866             Paolucci, R., Faccioli, E., Maggio, F., 1999. 3D Response analysis of an instrumented hill at Matsuzaki, Japan, by a

867	spectral method. J. Seismol. https://doi.org/10.1023/A:1009890320625

868       Paolucci, R., 2002. Amplification of earthquake ground motion by steep topographic irregularities. Earthquake  Engng.

869	Struct. Dyn., 31: 1831-1853. doi:10.1002/eqe.192

870	Peter, D., Komatitsch, D., Luo, Y., Martin, R., Le Goff, N., Casarotti, E., Le Loher, P., Magnoni, F., Liu, Q.,   Blitz, C., 871		Nissen-Meyer, T., Basini, P., Tromp, J., 2011. Forward and adjoint simulations of seismic wave propagation 872		on fully unstructured hexahedral meshes. Geophys. J. Int. https://doi.org/10.1111/j.1365-246X.2011.05044.x 873	Pischiutta, M., Cultrera, G., Caserta, A., Luzi, L. & Rovelli, A., 2010. Topographic effects on the hill of Nocera Umbra, 874		central Italy, Geophys. J. Int., 182, 977–987.

875       Pischiutta, M., Cultrera, G., Caserta, A., Luzi, L. & Rovelli, A., 2010. Topographic effects on the hill of Nocera Umbra,

876	central Italy, Geophys. J. Int., 182, 977–987.

877  Pischiutta, M., Cianfarra, P., Salvini, F., Cara, F., & Vannoli, P., 2018. A systematic analysis of directional site effects   878	at  stations  of  the  Italian  seismic  network  to   test  the   role  of  local       topography. Geophysical  Journal 879	International, 214(1), 635-650. DOI: 10.1093/gji/ggy133

880  Pizzi, A., Di Domenica, A., Gallovič, F., Luzi, L., Puglia, R., 2017. Fault Segmentation as Constraint to the Occurrence 881	of      the      Main      Shocks      of      the      2016      Central      Italy      Seismic        Sequence.      Tectonics. 882	https://doi.org/10.1002/2017TC004652

883     Puzzilli, L.M., Ferri, F., Eulilli, V., Calcaterra, S., Gambino, P., Porfidia, B., Roma, M., Bellaveglia, S., Bistocchi,      884	R.M., Gattoni, M., 2019. Integrated geophysical methods for the seismic site characterization of Arquata del 885	Tronto (AP), in: Earthquake Geotechnical Engineering for Protection and Development of Environment and 886	Constructions- Proceedings of the 7th International Conference on Earthquake Geotechnical Engineering.

887  Sepúlveda, S.A., Murphy, W., Jibson, R.W., Petley, D.N., 2005. Seismically induced rock slope failures resulting from  888	topographic amplification of strong ground motions: The case of Pacoima Canyon, California.    Eng Geol, 80: 889	336– 348

890              Seriani, G., Priolo, E., 1994. Spectral element method for acoustic  wave simulation in heterogeneous media. Finite

891	Elem. Anal. Des. https://doi.org/10.1016/0168-874X(94)90076-0

892       Sextos,  A., De Risi, R., Pagliaroli, A., Foti, S., Passeri, F., Ausilio, E., Cairo, R., Capatti, M.C., Chiabrando, F.,        893	Chiaradonna, A., Dashti, S., De Silva, F., Dezi, F., Durante, M.G., Giallini, S., Lanzo, G., Sica,   S., Simonelli, 894	A.L., Zimmaro, P., 2018. Local site effects and incremental damage of buildings during the 2016  Central Italy 895	Earthquake sequence. Earthq. Spectra. https://doi.org/10.1193/100317EQS194M

896  Spudich, P., Hellweg, M., Lee, W.H., 1996. Directional topographic site response at Tarzana observed in aftershocks of 897	the 1994 Northridge, California, earthquake: implication for mainshock motion, Bull. Seismol. Soc.   Am., 86, 898	193–208.

899          Tarquini, S., Isola, I., Favalli, M., Mazzarini, F., Bisson, M., Pareschi, M.T., Boschi, E., 2007. TINITALY/01: A new

900	Triangular Irregular Network of Italy. Ann. Geophys. https://doi.org/10.4401/ag-4424

901            Vessia, G., Di Curzio, D., Castrignanò, A., 2020a. Modeling 3D soil lithotypes variability through geostatistical data

902	fusion of CPT parameters. Sci. Total Environ. https://doi.org/10.1016/j.scitotenv.2019.134340

903  Vessia, G., Di Curzio, D., Chiaudani, A., Rusi, S., 2020b. Regional rainfall threshold maps drawn through multivariate  904	geostatistical      techniques      for      shallow      landslide      hazard      zonation.      Sci.      Total      Environ. 905	https://doi.org/10.1016/j.scitotenv.2019.135815

906               Vessia, G., Parise, M., Tromba, G., 2013. A strategy to address the task of seismic  microzoning in landslide-prone

907	areas. Adv. Geosci. https://doi.org/10.5194/adgeo-35-23-2013

908          Vessia, G., Pisano, L., Tromba, G., Parise, M., 2016. Seismically induced slope instability maps validated at an urban

909	scale by site numerical simulations. Bull. Eng. Geol. Environ. https://doi.org/10.1007/s10064-016-0940-0

910  Xie,  Z.,  Komatitsch,  D.,  Martin,  R.,  Matzen,  R.,  2014.  Improved  forward  wave  propagation  and  adjoint-based   911	sensitivity   kernel   calculations   using   a   numerically    stable   finite-element   PML.   Geophys.      J.   Int. 912	https://doi.org/10.1093/gji/ggu219

913              Zhang, Z., Fleurisson, J-A, Pellet F.L., 2018. A case study of site effects on seismic ground motions at Xishan Park

914	ridge in Zigong, Sichuan, China. Eng. Geol. 243: 308–319

915       APPENDIX A: Geological characters

916    The Apennines are a North and North-East verging fold-and-thrust belt, originated by the convergence and collision     917  between the African (Adria-Apulia) and European plates. The Central-Northern portion of this chain is a tectonic arch,  918    bordered in the South by the E-NE verging Olevano-Antrodoco-Sibillini Mountains thrust (OASt) (Pizzi et al., 2017    919    and references therein). During Early Pliocene age, the OASt caused the overlapping of the Umbria-Marche basinal     920   carbonate succession above the Latium-Abruzzi carbonate  platform in the South part and  above Messinian  foredeep   921  siliciclastic deposits of the Laga Basin and minor siliciclastic basins in the Northern Marche region, and above the Gran 922  Sasso  succession (Centamore  et  al.,  1991).  During Pleistocene,  the  activation of  normal  faults,  mainly NW-SE  or 923    NNW-SSE oriented, affected the inner portions of the thrust belt and conditioned the present-day geological setting.     924  Arquata  del Tronto  is located  on the  outer  edge  of the  Central  Apennines at the  foot of the  OASt,  in the  Western 925  outcropping area of the Messinian turbidite deposits of the Laga Formation. In this portion, this unit is also deformed by 926      thrusts and folds (Centamore  et al., 1991). The Laga  Formation was firstly subdivided into three  members: pre-       927     evaporitic, evaporitic and post-evaporitic, related to the main Messinian events of the Mediterranean region. Facies     928  analysis allowed interpreting the Laga Formation as a classical deep-sea fan turbidite succession (Milli et al., 2013 and  929   references herein). Subsequently, the formation is subdivided into sub-members characterized by different content of    930       arenite and pelite.

931       In the analysed area, four lithofacies associations of this member crop out.

932  The arenaceous association (LAG4c) consists of arenaceous layers from thick to very thick, blended or separated by thin 933   pelitic intercalations. The bodies have large-scale tabular and lenticular geometries, prevailing facies A1, A2, B1, B2,   934  C1  and  C2.  At  various  heights,  there  are  arenaceous-pelitic  and  subordinately  pelitic-arenaceous  non-mappable   935  horizons. The succession can be ascribed to channelled deposits of an internal fan. The arenaceous-pelitic association I  936   (LAG4d) consists of alternations of arenaceous horizons in very thick layers, in facies A1, B1 and C1 and arenaceous-   937      pelitic horizons in facies C2,  C1 and  D1. The A/P ratio is between 3 and 10. The surfacing unit has at least three       938    fracture  systems and is  sometimes altered on the surface.  The arenaceous-pelitic association II (LAG4b) consists of    939  medium to thick tabular beds with the lower portion graded and upper laminated, complete sequence of Bouma (facies   940  C and subordinately D1 and D2), ratio A/P between 1 and 3. The pelitic-arenaceous association (LAG4e) consists of thin 941  pelitic-arenaceous  layers  in  facies  D2  and  D3  with  intercalated,  at  different  heights,  arenaceous-pelitic  horizons 942    consisting of medium strata, in prevalent facies C2, C1, B1 and D1. The succession is attributable to non-channelled    943       deposits of an external fan.

944

945       APPENDIX B: Theoretical features of GeoModeller

946  GeoModeller, the package we adopt for the geological 3D modelling is, a commercial software originally developed by 947   the  French  Bureau  de   Recherches   Géologiques  et  Minières  (BRGM)  and   then   by  the  Intrepid   Geophysics    948  (http://www.geomodeller.com,  last  access:  27  November  2019, Calcagno  et  al.,  2008; Guillen et  al.,  2004). This   949  software  allows  building  complex  and  implicit  3D  geological  models  directly  from  geological  observations.  The 950  interpolation method is based on the potential field theory (Lajaunie et al., 1997), so that geological interfaces (i.e. the   951  upper or lower surfaces of the geological units) are modelled as iso-surfaces of a scalar potential field defined in the 3D 952  space. Structural data are treated as the gradient of the field. The interpolation of the field uses co-Kriging (Vessia et al., 953   2020a; 2020b and references therein) to take both contacts and structural data into account and generates surfaces that   954  honour all the data together (McInerney and Guillen, 2005). The adopted approach also employs rule-based modelling   955   to control the relationships in the stratigraphic pile (either ‘onlapping’ or ‘erosional’), and to control fault chronology    956  within the fault network (Chiles et al., 2007). All the details about the application of this method for building geological 957       models can be found in Calcagno et al. (2008).

958

959       APPENDIX C: 2D numerical seismic response method

960     2D numerical analyses have been performed using the commercial code LSR2D by Stacec s.r.l. (www.stacec.com).     961  This 2D code implements the finite element method that allows calculating stresses and deformations, under total stress 962  conditions, induced by the 2D propagation of horizontal shear waves SH and compressional waves P from the bedrock   963  throughout the modelled domain of soils and rocks. The code solves the general equation of the dynamic motion at each 964       node of the finite element mesh grid:

965    Mu.. + (Ce + Cb)u· + Ku = −MlX ub..,X(t) − Mlyub..,y(t) + Fff(t)	(C.1)

966     where: M = the global mass matrix of the equation system,   Ce   = global damping matrix of the  finite element node     967   system;  Cb   = global damping matrix of the viscous dampers at the bottom of the mesh; K = global stiffness matrix of    968    the equation system; u..  = global acceleration; vector, u·  = global velocity vector and u = global displacement vector of    969   the equation system; lX   = the drag global vector along horizontal direction;   ly   = the drag global vector along vertical    970     direction; u.. b,X,y   = horizontal and vertical components of the input acceleration vector; Fff(t) = dynamic forces that     971       simulate the free field conditions at the vertical boundaries of the modelled domain.

972   The behaviour of soils and rocks are represented by the equivalent linear model. Through this constitutive model, the    973  shear soil response to seismic solicitations is interpreted by an equivalent viscoelastic behaviour consisting on the shear 974    modulus G that is updated at each time step of the analysis according to the shear strain value γ induced by the input    975  solicitation. Its decreasing values are described by a curve measured from lab tests known as cyclic degradation curve.   976      Accordingly, to take into account the damping effects of the cyclic degradation, the damping curve D(γ) is used to      977    update the damping values to the shear strain level. The damping curve is derived by lab tests, too. Several curves of    978    G(γ) and D(γ) from literature are available and often used for such numerical simulations on soils according to some    979    physical parameters. In the case of rocks, the stiffness of the materials is much higher than the soil resistance and the    980  hypothesis  of  very small  shear  strain  deformations  induced  by seismic  shaking  is  considered  verified,  the  elastic 981      behaviour is selected. This latter consists of fixed values of G and D assumed during the input motion.

- The global damping matrix, Ce, it is obtained by assembling the finite element damping values through a classical

- Rayleigh scheme. For each element I, the damping can be written as:

- Ci  = aR,iMi  + /3R,iKi	(C.2)

- where  aR,i   and  /3R,i   are  the  Rayleigh  coefficients  and  Mi   and  Ki   are  the  mass  and  stiffness  matrices  of  each  finite

- element, respectively.

- The whole domain has been divided into triangular or quadrangular elements  which maximum side dimension is      988   decided according to the cut off frequency, that is the maximum propagated frequency fmax. Commonly, for near-field   989  sites, fmax = 25 Hz is assigned. Then, to avoid the numerical aliasing phenomenon, the following rule has been adopting  990       for the maximum element side dimension (Kuhlemeyer and Lysmer, 1973):

991	ℎ = 	VS

6÷8∙fmax

(B.3)

992            The input motion (see 4.1.2.) is given at the bottom of the model domain, and results, calculated in the time domain,

993  have been reported in terms of acceleration spectral responses and the acceleration amplification function. This latter is  994   the ratio, in the period/frequency domain, between the output response spectrum at a point on the upper surface of the   995    modelled domain and the input response spectrum applied at the bottom of the model, representing the bedrock line.    996  Finally, lateral boundary conditions have been adopted to simulate the absorption of the wave associated energy by the  997  semi-infinite domain. This type of damping edge conditions was introduced as free-field columns, corresponding to the  998       equations:

999     FX    = −pVP(u· m − u· ff)A	(C.4)

X	X

1000

Fy  = −pVS(u· m − u· ff)A	(C.5)

y	y

1001

1002

1003

where: p= soil mass density of the soil; VP: P wave velocity of the soil; VS: S wave velocity of the soil; A: the influence area of the damping node;  u ̇ m: node velocity in x-direction;  u ̇ m: node velocity in y-direction; u ̇ ff: node velocity of the free field column in x-direction; u ̇ ff: node velocity of the free field column in the y-direction.